| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS – O3 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 00 – 19 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Dust Mitigation | Page Number |
|  |  | Page 1 of 3 |

| Date | 19 August 2024 |
| --- | --- |
| Site Name: | The Rugby Village, Portion 63 & 25 Farm Gustrouw 918, Mondeor Road, Stellenbosch Farms, City of Cape Town, 7130 |

Activity: Dust Mitigation

- Dust mitigation

- Stockpile requirements

- Plant and traffic rules

- Water restrictions/water usage

SCOPE

The method statement will outline the required management controls that will be implemented on site to control excessive dust created by both construction activities and inclement weather

INTRODUCTION

WORKS, MANAGEMENT ACTIONS, CONTROLS

# Dust management

  - Plant and vehicles shall at all times comply with implemented 20Km/h speed limits. Refer to site Traffic management plan for more information.

  - Loads of delivery vehicles (e.g. bricks, sand, cement, etc.) shall be suitably covered and secured during transportation.

  - During high wind conditions, the Site Manager (Mynhardt Enslin) within consultation with the Site Foreman (Calie Vermeulen) evaluate the situation and make recommendations as whether dust-damping measures are adequate, or need improvement.

  - Rhinos Rugby Academy shall implement dust suppression measures (e.g. covering of material stockpiles, chemical soil binders, shade cloth erected on fencing etc.) if and when required. Straw, and chipping shall be considered prior to the use of water spray vehicles.

  - Water from the dams located west side of the site/ boreholes will be used to suppress dust on roads in hot & dry days where there is a possibility of vehicles creating dust on site.

  - Plastic, shade cloth or other suitable material may be used for covering stockpiles.

  - Topsoil stockpiles will not to be covered with plastic or shade cloth but hessian or similar to prevent composting occurring.

  - The Civil Contractor and Rhinos Rugby Academy will ensure that the generation of dust is minimised and will implement a dust control programme to maintain a safe working environment, minimise nuisance for surrounding residential areas/dwellings and protect damage to natural vegetation.

  - Retain vegetation where possible to minimise excessive dust

  - Straw stabilization may be used in approved areas within the site once site clearing has commenced.

  - Drinking and Municipal water may not be used for dust suppression, only waste, recycled or borehole water may be used.

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS – O3 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 00 – 19 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Dust Mitigation | Page Number |
|  |  | Page 2 of 3 |

Annexure 1 (water for dust suppression)

# The following will be relevant contact details:

| Name and Surname | Representing – Position | Contract details |
| --- | --- | --- |
| Mynhardt Enslin | Rhinos Rugby Academy- Project Director/Site Manager | 083 603 0770 |
| Neels Van Koeler | Boland Grondwerk – Civil Contractor | 082 940 1749 |
| Calie Vermeulen | Boland Grondwerk – Site Foreman | 083 607 8589 |
| Chris Oosthuizen | Safe Working Practice – SHEQ Consultant | 082 888 6456 |
| Monique Norman | Health & Safety Manager | 072 261 8203 |
| Klye Enslin | Pepper Pod - Construction Manager | 083 759 5816 |
| Roelof Rabe | Roelof Rabe – Consulting Architect | 021 914 1690 |
| Divan Opperman | Guillaume Nel Environmental Consultants | 076 453 8829 |

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS – O3 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 00 – 19 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Dust Mitigation | Page Number |
|  |  | Page 3 of 3 |

DECLARATIONS FOR ENVIRONMENTAL METHOD STATEMENT

| SITE MANAGER The works described in this Method Statement, if carried out according to the methodology described, is satisfactory to prevent or control environmental harm and is thus approved: SM Name Approval Date Signature | CONTRACTOR I understand the contents of this Method Statement and the scope of the works required of me. I further understand that this Method Statement may be amended on application to and with approval by the Site Manager, and that the SHE Officer, Construction Manager and ECO will audit my compliance with the contents of this Method Statement: Contractor Acceptance Date Signature |
| --- | --- |
| ENVIRONMENTAL CONTROL OFFICER () The works described in this Method Statement, if carried out according to the methodology described, is satisfactory to prevent or control environmental harm and is thus approved: Name Approval Date Signature; COMMENTS:. |  |