| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS – 09 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 00 – 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Topsoil management | Page Number |
|  |  | Page 1 of 1 |

| Date | 23 August 2024 |
| --- | --- |
| Site Name: | The Rugby Village, Portion 63 & 25 Farm Gustrouw 918, Mondeor Road, Stellenbosch Farms, City of Cape Town, 7130 |

# Activity: Top soil management

- Vegetation and topsoil clearing

SCOPE

This method statement outlines the methods that will be used to carry out vegetation/top soil clearing

INTRODUCTION

WORKS, MANAGEMENT ACTIONS, CONTROLS

How the works will be undertaken:

Topsoil/top materials shall be removed from all area cleared of vegetation and be retained for future landscaping use, where feasible.  Top material should exclude litter, building rubble , alien plant material or other waste.

Topsoil shall be stored in areas demarcated by the ESM and Site Manager and in piles not higher than 2m.  The stockpiles shall not be compacted or disturbed and shall be domed at the top to promote runoff.  Should significant erosion (eg through rain and wind) of the stockpiled material occur, the stockpiles will be covered with vegetation cleared from the site.

MONITORING AND RECORD KEEPING

(Description of  how the activity will be monitored to ensure that the environmental standards are met, as well as the records to be kept)

- Daily site walkabout by Site Manager	Record	Site diary entry

- Weekly inspection by Contractor 		Record	Weekly checklist

- Monthly audits by ECO/Employer EO	Record	Monthly compliance audit checklist

- Incident reporting				Record	Environmental Incident reports 									including corrective action

- Staff training					Record	Induction training register, toolbox

Talks attendance register

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS – O9 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 00 – 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT TopSoil Management | Page Number |
|  |  | Page 2 of 3 |

| Name and Surname | Representing – Position | Contract details |
| --- | --- | --- |
| Mynhardt Enslin | Rhinos Rugby Academy- Project Director/Site Manager | 083 603 0770 |
| Neels Van Koeler | Boland Grondwerk – Civil Contractor | 082 940 1749 |
| Calie Vermeulen | Boland Grondwerk – Site Foreman | 083 607 8589 |
| Chris Oosthuizen | Safe Working Practice – SHEQ Consultant | 082 888 6456 |
| Monique Norman | Health & Safety Manager | 072 261 8203 |
| Klye Enslin | Pepper Pod - Construction Manager | 083 759 5816 |
| Roelof Rabe | Roelof Rabe – Consulting Architect | 021 914 1690 |
| Divan Opperman | Guillaume Nel Environmental Consultants | 076 453 8829 |

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS – O9 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 00 – 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT TopSoil Management | Page Number |
|  |  | Page 3 of 3 |

DECLARATIONS FOR ENVIRONMENTAL METHOD STATEMENT

| SITE MANAGER The works described in this Method Statement, if carried out according to the methodology described, is satisfactory to prevent or control environmental harm and is thus approved: SM Name Approval Date Signature | CONTRACTOR I understand the contents of this Method Statement and the scope of the works required of me. I further understand that this Method Statement may be amended on application to and with approval by the Site Manager, and that the SHE Officer, Construction Manager and ECO will audit my compliance with the contents of this Method Statement: Contractor ___________ _ __ Acceptance Date ___________ _____ Signature ______________ __ |
| --- | --- |
| ENVIRONMENTAL CONTROL OFFICER (ECO) The works described in this Method Statement, if carried out according to the methodology described, is satisfactory to prevent or control environmental harm and is thus approved: ECO Name Approval Date Signature | COMMENTS: |