| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS11 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Traffic Management | Page Number |
|  |  | Page 1 of 6 |

| Date: | 23 August 2024 |
| --- | --- |
| Site Name: | The Rugby Village, Portion 63 &25 Farm Gustrouw 918, Mondeor Road, Stellenbosch Farms, City of Cape Town, 7130 |

Activity:  Traffic Accommodation

- Detailed method of how traffic will be managed on site with layouts.

- Parking areas

SCOPE

INTRODUCTION

- Traffic accommodation for road works on site provide means of planning and implementing how likely road users will be safely and efficiently guided through road works site and to ensure the network performance is not unduly impacted, for the duration of the works TMP to be maintained as the work progresses.

WORKS, MANAGEMENT ACTIONS, CONTROLS

# Purpose of the Traffic & Transportation Management Plan

Traffic accommodation for road works on site provide means of planning and implementing how likely road users will be safely and efficiently guided through a road works site and to ensure the network performance is not unduly impacted, for the duration of the works TMP to be maintained as the work progresses.

  - Traffic control drawing outlining signage and device placement which are commonly used.

  - Inspection of the site at least once prior to preparation of the TMP. The site inspection is required to identify specific site conditions, vulnerable road user facilities, existing signs, lighting, adjoining road networks and other possible constraints.

Both traffic and transportation go hand in hand, thus ensuring that traffic and the transportation of workers/equipment are effectively controlled and maintained in an appropriate manner.

The traffic accommodation will implement measures to ensure compliance in accordance with the EMPr, HSE Plans, Policies & Specifications.

Details of work:

Working hours for this project are from the 7h00am till 17h00pm. Current speed limit is 20km/h for all vehicles.

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS11 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Traffic Management | Page Number |
|  |  | Page 2 of 6 |

# Proposed location

The proposed location of the site The Rugby Village, Mondeor Road,

Helderberg Rural.

# Roles & Responsibilities

  - Plant and equipment – all equipment must have reversing alarms to warn motorists and personnel on foot of their presence. For more information refer to the construction traffic and access procedure.

  - Incident/Accident procedures – outlining first aid facilities, arrangements for obtaining medical assistance and the requirements for reporting accidents. For more information refer to the Emergency preparedness and response procedure and Incident reporting procedure.

- Construction Manager:

The Construction Manager will have the responsibility to effectively co-ordinate planning, manpower, equipment allocations and health and safety, to ensure on-time completion of planned work.

The Construction manager will take overall responsibility of the plan,compliance with regards to the Road Traffic Act and Traffic management Plan. The Construction Manager will have overall responsibility for ensuring that the functions defined in this Traffic management Plan are carried out effectively.

Ensure all employees comply with the Traffic management Plan.

- SHEQ Officer:

The SHEQ Officer will be responsible for all issues related to health, safety,and environment and to see that employees conforms to the requirements as laid down by the Occupational Health & Safety Act and National Environmental Management Act, and/or those acts applicable within South Africa.

He/she will conduct regular toolbox talks, maintain relevant registers, ensure that the relevant inspections relating to the Traffic Accommodation are compiled, and distributed to those responsible. The SHEQ Officer will ensure that all SWP and RA of the Traffic Accommodation are discussed beforehand with all relevant people and proof of communication will be kept in the SHEQ File for inspection purposes. Traffic management layout plan will form part of the induction process and will be displayed in the site office and on the notice board.

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS11 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Traffic Management | Page Number |
|  |  | Page 3 of 6 |

- References: Legal and other provisions

The current versions of the following documents and legislative provisions apply for those planning to conduct work within the road reserve of any public road, or to manage traffic for a construction activity in specific areas:

  - National Road Traffic Act (Act No 93 of 1996)

  - Local Government Act

  - Occupational Health & Safety Act (Act 85 of 1993)

  - Construction Regulations 2014

  - Client Specification

  - EMPr

# Risk Identification and Assessment

Risk Assessments will be drafted beforehand including all possible risks that might occur due to traffic and transportation management.

The risk assessors shall as far as reasonably practicable, control or reduce the risks to a manageable manner. Factors such as speeding, the transportation of loads, the use of public transport/company transport must be properly assessed to ensure that incidents are minimized to a lesser extent.

# General traffic and transportation

All personnel will be advised to stay clear of any vehicles. All vehicles have the right of way. Operators should also be always aware of their surroundings. Pedestrians are to walk on the side of the road to avoid construction traffic.

Access control will be managed at the gate to ensure that no non authorized person enters.

Rules for vehicle safety

  - Personnel must be trained, declared competent and authorized to operate or drive any.  Vehicle as required under the national road traffic act.

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS11 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Traffic Management | Page Number |
|  |  | Page 4 of 6 |

  - The vehicle must be in a safe, working condition.

  - Drivers/Operators must always consider and adapt to environment and weather.

  - Drivers/Operators must always comply with all relevant traffic rules and regulations as per induction training.

  - Seatbelts shall be worn when driving or operating vehicles or plant fitted with seatbelts .

  - Drivers and operators shall comply with relevant speed limits. (20 km/h on site, applicable site speeding rules)

  - No talking on cell phone while driving.

  - Adhere to all site traffic rules and signage.

  - Driving under the influence of alcohol or drugs is prohibited as per drugs and alcohol policy.

  - Daily inspections of vehicles must be conducted as per construction regulations 23.

# Emergency planning:

Contact details of The Rugby Village and emergency services will be conveyed to all necessary personnel and workforce, thus ensuring that in the event of any incident occurring, the necessary services are informed immediately. Refer to the The Rugby Village  Emergency Evacuation Plan.

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS11 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Traffic Management | Page Number |
|  |  | Page 5 of 6 |

ANNEXURE 1:  ACCESS ROAD AND LOCATIONS and DETAIL OF CAMP AREA ESTABLISHMENT

| Name and Surname | Representing – Position | Contract details |
| --- | --- | --- |
| Mynhardt Enslin | Rhinos Rugby Academy- Project Director/Site Manager | 083 603 0770 |
| Neels Van Koeler | Boland Grondwerk – Civil Contractor | 082 940 1749 |
| Calie Vermeulen | Boland Grondwerk – Site Foreman | 083 607 8589 |
| Chris Oosthuizen | Safe Working Practice – SHEQ Consultant | 082 888 6456 |
| Monique Norman | Health & Safety Manager | 072 261 8203 |
| Klye Enslin | Pepper Pod - Construction Manager | 083 759 5816 |
| Roelof Rabe | Roelof Rabe – Consulting Architect | 021 914 1690 |
| Divan Opperman | Guillaume Nel Environmental Consultants | 076 453 8829 |

| Rhinos Asset Holdings | The Rugby Village | Document Number |
| --- | --- | --- |
|  |  | EMS11 |
|  | Integrated Management System | Revision No. & Date |
|  |  | 23 August 2024 |
|  | ENVIRONMENTAL METHOD STATEMENT Traffic Management | Page Number |
|  |  | Page 6 of 6 |

DECLARATIONS FOR ENVIRONMENTAL METHOD STATEMENT

| CONSTRUCTION MANAGER The works described in this Method Statement, if | CONTRACTOR I understand the contents of this Method Statement |
| --- | --- |
| carried out according to the methodology | and the scope of the works required of me. I further |
| described, is satisfactory to prevent or control environmental harm and is thus approved: | understand that this Method Statement may be amended on application to and with approval by |
|  | the Engineer, and that the SHEQ Officer, |
| Construction manager : ___ _ | Construction Manager and ECO will audit my compliance with the contents of this Method Statement: |
| Approval Date: | Contractor |
| Signature: ___ _ | Acceptance Date |
|  | Signature |
| ENVIRONMENTAL CONTROL OFFICER (ECO) The works described in this Method Statement, if carried out according to the methodology described, is satisfactory to prevent or control environmental harm and is thus approved: ECO Name Approval Date Signature | COMMENTS: |